STANDARD OPERATING PROCEDURE (SOP)
Title: Equipment Cleaning and Preventive Maintenance
SOP No.: SOP/PHAR/007
Version: 1.0
Effective Date: 09-Dec-2025
Prepared by: Mr. Rohit Verma, QA Executive
Reviewed by: Ms. Shilpa Rao, QA Head
Approved by: Ms. Priya Kapoor, Plant Director

### 1. Purpose

The purpose of this SOP is to provide standardized procedures for cleaning, sanitizing, and maintaining pharmaceutical manufacturing and laboratory equipment to ensure product quality, prevent contamination, and comply with Good Manufacturing Practices (GMP).

### 2. Scope

This SOP applies to all equipment used in manufacturing, packaging, and quality control laboratories at SunPharma Industries Pvt. Ltd., Mumbai. It includes cleaning, routine maintenance, and preventive maintenance schedules.

### 3. Responsibility

- Operators / Technicians: Perform routine cleaning and daily maintenance.

  - Example: Mr. Arjun Singh (Production Line 2), Ms. Neha Desai (QC Lab)

- Maintenance Team: Conduct preventive maintenance, repairs, and calibration of equipment.

  - Example: Mr. Ramesh Patil, Mr. Sunil Kulkarni

- QA Personnel: Verify cleaning logs, approve maintenance activities, and ensure compliance.

  - Example: Ms. Shilpa Rao

### 4. Definitions

- Cleaning: Physical removal of visible residues from equipment surfaces.

- Sanitization / Disinfection: Reduction of microbial contamination to acceptable levels.

- Preventive Maintenance (PM): Scheduled inspection, lubrication, calibration, and replacement of parts to avoid breakdowns.

- Critical Equipment: Equipment that directly affects product quality (e.g., tablet press, HPLC, sterilizers).

### 5. Procedure

5.1 Equipment Cleaning

- Preparation:

  - Wear appropriate PPE (gloves, gown, mask, goggles).

  - Ensure equipment is turned off and disconnected from power sources.

- Cleaning Steps:

  - Remove detachable parts and clean with approved detergent solution.

  - Wipe all surfaces with 70% IPA or other validated disinfectant.

  - Rinse with purified water if necessary and allow to dry.

  - Reassemble equipment.

- Frequency:

  - Daily: After production or testing cycle.

  - Weekly: Deep cleaning of critical surfaces and hard-to-reach areas.

Example Log Entry:

| Date | Equipment | Cleaning Type | Performed By | Verified By |
| --- | --- | --- | --- | --- |
| 09-Dec-2025 | Tablet Press T-12 | Routine | Mr. Arjun Singh | Ms. Shilpa Rao |

5.2 Preventive Maintenance (PM)

- Schedule:

  - Monthly: Minor inspection, lubrication, and adjustment of moving parts.

  - Quarterly: Calibration verification and replacement of worn-out components.

  - Annually: Full maintenance and calibration check by external certified service.

- Procedure:

  - Inspect equipment for wear, leaks, corrosion, or unusual noise.

  - Lubricate moving parts using approved lubricants.

  - Replace seals, filters, and worn components.

  - Record maintenance details in Preventive Maintenance Log.

Example PM Log Entry:

| Date | Equipment | Activity | Performed By | Verified By |
| --- | --- | --- | --- | --- |
| 09-Dec-2025 | HPLC System H-04 | Monthly PM | Mr. Sunil Kulkarni | Ms. Shilpa Rao |

5.3 Documentation and Verification

- Maintain Cleaning Log Sheets and PM Logs for all equipment.

- QA personnel to review and sign off completed logs weekly.

- Retain records for a minimum of 3 years or as per regulatory requirements.

### 6. Safety Precautions

- Always wear PPE during cleaning and maintenance.

- Use only approved detergents and disinfectants.

- Ensure electrical equipment is properly shut down before maintenance.

- Avoid contact with sharp edges or moving parts.

### 7. References

- Good Manufacturing Practices (GMP) guidelines

- WHO Technical Report Series on Pharmaceutical Equipment Cleaning

- Manufacturer’s equipment manuals

### 8. Revision History

| Version | Date | Description | Prepared by | Approved by |
| --- | --- | --- | --- | --- |
| 1.0 | 09-Dec-2025 | Initial issue | Mr. Rohit Verma | Ms. Priya Kapoo |