STANDARD OPERATING PROCEDURE (SOP)
Title: Storage Conditions Monitoring for Raw Materials and Finished Products
SOP No.: SOP/PHAR/006
Version: 1.0
Effective Date: 09-Dec-2025
Prepared by: Mr. Rohit Verma, QA Executive
Reviewed by: Ms. Shilpa Rao, QA Head
Approved by: Ms. Priya Kapoor, Plant Director

### 1. Purpose

This SOP describes the procedures for monitoring and maintaining appropriate storage conditions (temperature, humidity, and cleanliness) for raw materials, in-process materials, and finished products to ensure product quality, stability, and compliance with GMP regulations.

### 2. Scope

This SOP applies to all storage areas in SunPharma Industries Pvt. Ltd., Mumbai, including warehouses, cold rooms, controlled rooms, and finished goods storage. It covers all pharmaceutical raw materials, intermediates, and finished products.

### 3. Responsibility

- Warehouse Staff: Maintain storage conditions, record readings, report deviations.

  - Example: Mr. Anil Deshmukh, Ms. Kavita Joshi

- QA Personnel: Verify monitoring records, investigate deviations, approve corrective actions.

  - Example: Ms. Shilpa Rao, Mr. Rakesh Sharma

- EHS Officer: Ensure environmental monitoring equipment is calibrated and functional.

  - Example: Dr. Ananya Mehta

### 4. Definitions

- Controlled Room: Area where temperature and humidity are actively maintained within specified ranges.

- Temperature & Humidity Log: Record sheet for documenting environmental conditions.

- Deviation: Any reading outside specified storage conditions.

### 5. Procedure

5.1 Storage Condition Requirements

| Material Type | Temperature Range | Humidity Range | Notes |
| --- | --- | --- | --- |
| Raw Materials (APIs) | 20–25°C | 30–50% RH | Store away from sunlight |
| Excipients | 15–30°C | 40–60% RH | Ensure airtight containers |
| Finished Products | 20–25°C | 30–50% RH | FIFO / FEFO system |
| Cold Chain Products | 2–8°C | N/A | Refrigerators monitored |

5.2 Monitoring Procedure

- Frequency:

  - Warehouse: Twice daily (09:00 and 17:00)

  - Cold rooms: Continuous monitoring with data logger

- Instruments:

  - Thermometers, hygrometers, data loggers, and calibrated sensors

  - Ensure calibration records are up to date

- Recording Readings:

  - Fill Storage Condition Log Sheet immediately after reading

  - Example Entry:

| Date | Time | Location | Temp (°C) | Humidity (%) | Recorded by |
| --- | --- | --- | --- | --- | --- |
| 09-Dec-2025 | 09:00 | Warehouse A | 24 | 45 | Mr. Anil Deshmukh |

5.3 Handling Deviations

- Report immediately to QA and EHS personnel.

- Quarantine affected materials if required.

- Investigate cause and implement corrective actions.

- Record all actions in Deviation Report.

5.4 Equipment Maintenance

- Calibrate thermometers and hygrometers every 6 months.

- Replace sensors showing abnormal readings.

- Maintain calibration log with details: date, technician, instrument ID.

5.5 Documentation

- Storage Condition Log Sheet (daily readings)

- Calibration Log (instruments)

- Deviation Reports (if any)

- Retain all records for a minimum of 3 years or as per regulatory requirement

### 6. Safety Precautions

- Avoid opening cold rooms excessively to maintain temperature stability.

- Use PPE when accessing chemical storage areas.

- Ensure fire safety compliance in all storage areas.

### 7. References

- Good Manufacturing Practices (GMP) guidelines

- WHO Technical Report Series on Pharmaceutical Storage

- Local regulatory guidelines for pharmaceutical storage

### 8. Revision History

| Version | Date | Description | Prepared by | Approved by |
| --- | --- | --- | --- | --- |
| 1.0 | 09-Dec-2025 | Initial issue | Mr. Rohit Verma | Ms. Priya Kapoor |